AVR-7 / 9 Maestro M5/M9 Firmware update:

Please note: This update clears the Dirac calibration if run previously. Make sure you have access to your previous Dirac calibration file and upload once you have run the set up.

Step 1: check current firmware version:

Press the FRONT (INFO+DIRECT) Key for 3 sec

Step 2: Place the current “AC_System” file and “AC_Network” file on a USB drive.  Note that the “AC_System” file is not model specific, but the “AC_Network” file is specific.  Be sure no other versions of the firmware are on the USB drive.

Step 3: Connect the USB drive to the AVR-7 / 9 or Maestro M9.

Step 4: Press the FRONT (INFO + DISPLAY) Key for 3 sec until you see the AVR-7 /9 or Maestro M9 begin to load firmware.

Step 5: Wait until the AVR-7 /9 or Maestro M9 display reads “press OK”

Step 6: Press “OK” on the remote or “Select” on the front panel.  At this point the AVR-7 / 9 or Maestro M9 should reset.

Step 7: check that the firmware updated correctly

Press the FRONT (INFO+DIRECT) Key for 3 sec

Step 8: remove the USB drive from the AVR-7 / 9 or Maestro M9

Step 9: Reset to factory defaults – hold down the Menu button on the front panel for 6 seconds, then press Select, then press Direct. The unit will power cycle. You can then load your saved settings and then connect your PC and upload your Dirac calibration via the Dirac software.

-BDC  2017 OCT